# Improved Quadrotor Controller — Parameters and Formulas

Project: Autonomous drone navigation, PFE, Laboratoire DRIVE - Compétence SIC. Scope: the improved spiral-tracking controller (acceleration feedforward + thrust tilt-compensation + exact tilt inversion + tuned gains, under realistic saturation). Trajectory fixed at R=10 m, ω=0.6 rad/s.

Each parameter and formula is defined in plain language, with the symbol shown as a proper equation.

# 1. Fixed physical parameters

These describe the drone itself and never change (Singha et al., 2024).

- Mass m=1.80 kg — how heavy the drone is.

- Gravity g=9.81 m/s2 — the downward pull of the Earth.

- Arm length l=0.20 m — distance from the centre to each motor.

- Moments of inertia Ix=Iy=0.03, Iz=0.04 kg·m2 — how hard it is to spin the drone about each axis.

# 2. The 12 states (what describes the drone now)

x=[ x, y, z, vx, vy, vz, ϕ, θ, ψ, p, q, r ]T

- Position x,y,z (m): side, forward, up.

- Velocity vx,vy,vz (m/s): how fast it moves in each direction.

- Angles ϕ (roll), θ (pitch), ψ (yaw), in rad: how it is oriented.

- Body rates p,q,r (rad/s): how fast the angles change.

# 3. The four control inputs

- U1 — total thrust (all rotors together), sets height and, when tilted, sideways motion.

- U2 — roll torque (twist tipping left/right).

- U3 — pitch torque (twist tipping nose up/down).

- U4 — yaw torque (twist turning the nose). A torque is a twisting push.

# 4. Trajectory parameters and reference signals

- Radius R=10 m; angular rate ω=0.6 rad/s; climb rate vz=2 m/s; desired yaw ψd=π/4=45∘ (held constant).

Desired position, velocity and acceleration at time t:

xd=Rsin(ωt),  yd=Rcos(ωt),  zd=vz t

xd=Rωcos(ωt), yd=−Rωsin(ωt), zd=vz

xd=−Rω2sin(ωt), yd=−Rω2cos(ωt), zd=0

# 5. Controller gains

A gain decides how strongly the controller reacts to an error (error = wanted − actual). P reacts to how far off you are now; D reacts to how fast the error changes (a brake / damping).

- Horizontal: KP,xy=4.5, KD,xy=5.0.

- Height: KP,z=6, KD,z=5.

- Tilt (attitude): KP,att=25, KD,att=8.

- Yaw: KP,ψ=2, KD,ψ=2.

These were chosen by a gain sweep (grid search), not guessed.

# 6. Saturation (realistic command limits)

U1∈[ 1, 35 ] N,  |ϕd|≤30∘, |θd|≤30∘,  U2, U3, U4∈[ −5, 5 ]

Saturation means never asking for more than the real drone can give.

# 7. The recipe (formulas, in order)

Step A — errors. Position error =pd−p; velocity error =pd−p for each axis.

Step B — wanted horizontal acceleration (feedforward + PD):

ax=xd+KP,xy (xd−x)+KD,xy (xd−vx)

ay=yd+KP,xy (yd−y)+KD,xy (yd−vy)

The first term is the path’s own acceleration (feedforward, planning ahead); the other two fix position and speed.

Step C — thrust with tilt-compensation:

U1=mg+zd+KP,z (zd−z)+KD,z (zd−vz)cosϕ cosθ

Dividing by cosϕ cosθ compensates for the fact that a leaning drone sends less thrust upward. Then U1 is clipped into [1,35].

Step D — turn wanted sideways acceleration into a wanted lean. First scale to a lean fraction, then invert the geometry:

Uex=mU1 ax,  Uey=mU1 ay

ϕd=arcsin​Uexsinψ−Ueycosψ

θd=arcsin​Uexcosψ+Ueysinψcosϕd

The arcsin inputs are clamped to [−1,1] so they can never break (the bug fix). Then ϕd,θd are clipped to ±30∘. In short: wanted sideways push → required lean angles.

Step E — make the drone reach that lean (torques):

U2=IxlKP,att(ϕd−ϕ)−KD,att p−q r Iy−IzIx

U3=IylKP,att(θd−θ)−KD,att q−p r Iz−IxIy

U4=IzKP,ψ(ψd−ψ)−KD,ψ r−p q Ix−IyIz

Each line: push toward the wanted angle (P), brake the spin (D), and subtract a small spin-coupling correction (the qr, pr, pq terms). Then each of U2,U3,U4 is clipped to [−5,5].

# 8. Metrics

RMSE=1Nk=1Npk−pd,k2

- RMSE (m): the typical distance between where the drone is and where it should be.

- nRMSE: RMSE divided by the size of the motion, as a percent, for fair comparison across paths.

- Steady state: the first 5 s (the launch transient) are skipped; only the calm, repeating part is scored.

# 9. Clarifications

Tilt vs yaw are different angles. Yaw ψ=45∘ is the heading — which way the nose points (rotation about the vertical axis); the drone can hold 45∘ of yaw while perfectly flat. Tilt is the lean from horizontal (roll ϕ and pitch θ) and is what pushes the drone sideways; this is what we cap at 30∘. They are independent, so there is no conflict.

Origin of the formulas. The Step B law (feedforward + PD) is the standard trajectory-tracking law and matches Prof. Chaibet’s controller form. The Step C thrust law comes from inverting the drone’s own vertical dynamics:

z=cosϕ cosθm U1−g ⇒ U1=mzdes+gcosϕ cosθ

The Step D inversion is derived from the acceleration equations by rotating them by the yaw angle; it is exact and more accurate than the original code (which had an arcsin domain error) and the small-angle version.

# 10. Result summary (R=10, ω=0.6)

Steady-state position tracking improved from about 5 m (original) to about 0.18 m (improved), at peak tilt ≈34∘ and peak thrust ≈21 N — both within realistic limits. This is controller quality (desired vs actual), separate from the PINN observer.

# 11. How each parameter was determined

Not every number is tuned. They fall into three groups: fixed physical facts, task-defined choices, and values found by experiment.

Fixed physical parameters (m,l,g,Ix,Iy,Iz). These are not tuned at all — they are the drone’s specification from Singha et al. (2024). They enter the equations of motion as given constants.

Task-defined trajectory parameters (R,ω,vz,ψd). These define the flight we must perform (the Singha spiral) and were kept fixed on purpose (R=10, ω=0.6) so that improvements come from the controller, not from making the path easier.

Saturation limits. Chosen from physics, not tuned: max thrust 35 N from a realistic thrust-to-weight ratio of about 2 (hover needs mg=17.7 N); tilt limit 30∘ is a standard flyable bound; torque limit ±5 is consistent with the differential thrust a 35 N stack produces over a 0.20 m arm.

Exact tilt inversion (Step D). Not tuned — derived analytically by rotating the acceleration equations by the yaw angle. It replaced an approximate formula that could produce an out-of-range arcsin.

Steady-state window (skip 5 s). Chosen because the drone starts at (4, 5, 0), offset from the spiral’s position at t=0, so the first few seconds are a large catch-up transient; 5 s comfortably covers it, leaving only the repeating tracking regime to be scored.

Controller gains — found by a gain sweep (grid search). This is the only group that was truly tuned. For each candidate pair (KP,xy,KD,xy) we ran the full closed-loop simulation, measured the steady-state position RMSE under saturation, and kept the lowest feasible value. The resulting grid (steady overall RMSE in metres) was:

| KP,xy KD,xy | 3.0 | 4.0 | 5.0 |
| --- | --- | --- | --- |
| 1.0 | 0.460 | 0.397 | 0.484 |
| 1.5 | 0.381 | 0.284 | 0.290 |
| 3.0 | 0.259 | 0.228 | 0.207 |
| 4.5 | 0.514 | 0.185 | 0.180 |

The best feasible point was KP,xy=4.5, KD,xy=5.0 (steady RMSE ≈0.18 m, peak tilt ≈34∘, peak thrust ≈21 N — both within limits).

Two honest notes on this tuning. First, an earlier sweep (before the exact-inversion fix) wrongly suggested that gentle gains were best; that was an artifact of the arcsin bug, and once the inversion was fixed the expected trend returned (higher gains track better). Second, the attitude (inner-loop) gains KP,att=25, KD,att=8 are set deliberately high so the tilt loop is much faster than the position loop (time-scale separation), and the yaw and height gains are modest because yaw is held constant and height only climbs steadily. These were confirmed stable rather than exhaustively swept — a fuller multi-axis sweep is a possible refinement.

Important: all gains here were tuned on the spiral alone. Before trusting them generally, they should be checked on the circle and figure-8 references, since gains that suit one path can behave differently on another.